HUNTER AFB

ADJUSTABLE FLOOD BUBBLER

The pressure compensating bubbler shall have a full circle discharge rate of _________ GPM (m3/hr; l/m) at __________PSI (bars, kPa). The bubbler shall be constructed of corrosion and UV-resistant plastic, with an integral elastomeric flow bushing for maintaining a constant flow rate over the operating pressure range of 20 to 90 PSI (1.4 to 6.2 bars; 137 to 620 kPa). The maximum flow rate of the bubbler shall be 2 GPM (0.45 m3/hr, 7.6 l/m).

The bubbler shall be compatible with a plastic filter screen to protect the nozzle from debris in the water and a stainless steel adjustment screw that can be used to regulate the flow. The bubbler shall have a ½-inch Female National Pipe Thread (FNPT) inlet for connection to a ½-inch male threaded riser.

The nozzle shall carry a two-year, exchange warranty (not prorated).

The bubbler shall be manufactured for Hunter Industries Incorporated, San Marcos, California.