The valve shall be a normally closed, electronically-actuated, hydraulic, remote-control valve. The valve will be capable of operating between___________and_____________PSI (bars; kPa) with a flow range of between          ________and________GPM (m3/hr; l/m). Pressure loss shall be ____________PSI (bars; kPa) maximum at_______GPM (m3/hr; l/m).

The valve shall include an electric remote-control valve and atmospheric backflow preventer in one unit. The valve shall be equipped with a non-rising stem-type, manual flow control mechanism. This mechanism will be operable with a slotted screwdriver or with a removable flow-control handle. When specified for use with reclaimed water, a reclaimed water identifier handle shall be available.

The standard solenoid shall be a 24 VAC unit with a 350 mA inrush current and 190 mA holding current at 60 cycles and a 370 mA inrush current and 210 mA holding current at 50 cycles. When specified, the unit shall be equipped with a DC latching solenoid for use with 12-volt battery-operated controllers. The solenoid shall be an encapsulated, one-piece unit with captive plunger.  It shall be equipped with manual internal bleed capability to release the upper chamber water to the downstream piping, allowing the valve to open.  The valve shall have an internal manual bleed screw that provides an additional method for manual operation of the valve.

The body and bonnet shall be molded of non-corrodible PVC. The diaphragm shall be equipped with a serrated disc to screen water-borne contaminants, preventing them from reaching the metering area, and bleed ports of the valve. A perforated diaphragm support ring shall fit into the valve body just below the diaphragm to relieve stress on the diaphragm when the valve is closed.

The valve shall have a ¾-inch or 1-inch Female National Pipe Thread (FNPT) inlet and outlet. When specified, the valve shall have a ¾-inch or 1-inch slip inlet and outlet. All valve parts shall be serviceable after installation by unscrewing the bonnet screws, and removing the bonnet from the valve body to access the internal components. This may be accomplished without removing the valve body from the line.

The valve shall meet all IAPMO, ASSE 1001 and City of Los Angeles specifications.

The valve shall carry a two-year, exchange warranty (not prorated).

The valve shall be manufactured by Hunter Industries Incorporated, San Marcos, California.