Hunter Wind-Clik® Sensor Written Specifications

Part 1 – General

1.1  The Wind-Clik Sensor stops scheduled irrigation when it detects wind speed exceeding a preset rate by interrupting the power from the irrigation controller to the valves via a sensor terminal with wired communication.

Part 2 – Parts and Material

- All sensing components associated with the Wind-Clik Sensor shall be housed in a corrosion-resistant PVC casing.

Part 3 – Sensor Hardware

- Components

  - The wired Wind-Clik Sensor circuitry shall utilize a wind vane of 5" (13 cm) diameter to activate the switch in the unit for wind speeds in excess of the preset rate. The sensor can be set to actuate at various wind speeds from a minimum of 12 mph (19.3 km/hr) to a maximum of 35 mph (56.3 km/hr). The sensor shall have an adjustable reset for wind speeds of between 8 and 24 mph (12.8 to 38.6 km/hr).

  - The wired Wind-Clik Sensor shall utilize two knobs to set the actuation and reset wind speed rates. The large knob is the actuation speed in miles per hour. The small knob is the reset speed. This is the speed that the wind will have to slow down to in order to reset the switch. The difference between the two settings will determine the cycling of the system being controlled.

- Warranty

  - The Wind-Clik Sensor shall be installed in accordance with the manufacturer’s published instructions and shall carry a conditional five-year exchange warranty. The Wind-Clik Sensor shall be as manufactured for Hunter Industries Incorporated, San Marcos, California.

Part 4 – Operating Specifications

- Hardware

  - The wired Wind-Clik Sensor shall be mounted within 200' (60 m) of the irrigation controller.

  - The sensor shall have an integral mounting adapter that allows installation on a 2" (5 cm) PVC pipe or ½" (1 cm) conduit.

- General

  - The sensor shall be fully functional with 24 VAC input or less, with a current draw of 5 A, and shall not require a dedicated high-voltage transformer or connection.

© 2021 Hunter Industries™. Hunter, the Hunter logo, and all other trademarks are property of Hunter Industries, registered in the U.S. and other countries.