- I-90 Written Specifications

Part 1 – General

1.1  The I-90 is a long-distance rotary gear-driven sprinkler for natural turf, parks, sports fields, and public areas. It features a jar-top body cap that allows for quick access to the filter screen and check valve with no extra pieces, parts, or tools required.

Part 2 – Parts and Material

2.1  The I-90 shall be available in the following versions: 3" (8 cm) pop-up.

- The sprinkler shall be constructed from corrosion- and impact-resistant ABS plastic.

- The sprinkler shall have a ratcheting, stainless steel riser.

- The sprinkler shall be equipped with a durable, stainless steel riser spring.

- The sprinkler shall be available with an optional opposing nozzle housing.

- The sprinkler shall be available with 12 nozzle choices in flow rates from 29.5 to 83.8 GPM (6.7 to 19.04 m3/hr; 111.7 to 317.2 l/min).

- The arc of the sprinkler shall be adjustable from 40° to 360°.

- The sprinkler shall be equipped with through-the-top arc adjustment.

- The sprinkler shall be equipped with a QuickCheck™ arc mechanism.

- The sprinkler shall be equipped with a factory-installed rubber cover.

- The sprinkler shall be equipped with a standard drain check valve to hold back up to 9' 
(2.7 m) of elevation.

- The sprinkler shall be available in a reclaimed water ID version.

- The sprinkler shall be available with a user-installed rubber cover kit.

- The sprinkler shall be available with a user-installed turf cup.

2.2.  Rotor dimensions

A. I-90

    - Pop-up height: 3" (8 cm)

    - Overall height: 11" (28 cm)

    - Exposed diameter: 3½" (9 cm)

    - Inlet size: 1½" (40 mm) female NPT

2.3  Warranty

A.  The sprinkler shall carry a five-year exchange warranty.

Part 3 – Function and Operation

3.1  Recommended pressure range: 80 to 120 PSI (5.5 to 8.0 bar; 550 to 800 kPa)

A.  Operating pressure range: 70 to 120 PSI (5.0 to 8.0 bar; 500 to 800 kPa)

3.2  Radius: 73' to 104' (22.3 to 31.4 m)

3.3  Flow rate: 29.5 to 83.3 GPM (6.7 to 19.04 m3/hr; 111.7 to 317.2 l/min)

3.4  Precipitation rate: 0.75 in/hr (19 mm/hr) approximately

3.5  Nozzle trajectory: Standard: 22.5° approximately, Low-angle: 15° approximately

© 2019 Hunter Industries Inc. Hunter, the Hunter logo, and all other trademarks are property of Hunter Industries, registered in the U.S. and other countries.