- The Hunter MLD has an emitter welded to the inner circumference of the polyethylene tubing.  It is manufactured from high quality, UV-resistant resins to ensure both reliability and durability.  The dripline has a nominal size of .250” x .175” (6.4 mm x 4.4 mm) OD/ID.  It will accept standard ¼” (.18” or 4.6 mm) barb fittings.  It is offered in both brown and black with emitter spacing of every 6” or every 12”.  The MLD has two outlets per emitter, while specialized inlet filters reduce the chance of clogging.  The product is resistant to chemicals and fertilizers.  Flow ranges from .42 gallons per hour at 10 psi to .9 gallons per hour at 40 psi.  (1.6 liters per hour at .69 bar to 3.4 liters per hour at 2.76 bar).  Operating pressure is from 10-40 psi (.69 to 2.76 bar, 69 to 276 kPa)

- Available Models in Black:

- MLD0506100BL (6-inch emitter spacing, 100’ coil)

- MLD0506250BL (6-inch emitter spacing, 250’ coil)

- MLD0512100BL (12-inch emitter spacing, 100’ coil)

- MLD0512250BL (12-inch emitter spacing, 250’ coil)

- Available Models in Brown:

- MLD0506100 (6-inch emitter spacing, 100’ coil)

- MLD0506250 (6-inch emitter spacing, 250’ coil)

- MLD0512100 (12-inch emitter spacing, 100’ coil)

- MLD0512250 (12-inch emitter spacing, 250’ coil)

- MLD carries a 2-year warranty.