- Hunter Multi-port Emitters provide consistent, pressure-compensated flows directly to the root zone material.  Each emitter has six ports of equal flows.  Emitters come in four standard flow rates – 0.5, 1.0, 2.0, and 4.0 gallons per hour (2, 4, 8, 15 liters per hour). Each emitter is color-coded to indicate flow: blue is 0.5 gph (2 lph), black is 1.0 gph (4 lph), red is 2.0 gph (8 lph), and tan is 4.0 gph (15 lph). External surfaces are constructed with UV-resistant PVC material.  Each emitter port has its own diaphragm, made of molded silicone, allowing for greater consistency than one standard large diaphragm.  This emitter body is also offered without diaphragms having unrestricted flow for the use of being a manifold (MPM-050).  This manifold is color-coded in gray to indicate unrestricted flow.  Rubber MPE Caps may be used to cap off any unused ports.  Operating pressure of 5-65 psi (.34 to 4.5 bar, 34 to 448 kPa.)

- Available Models:

        - MPE-05

        - MPE-10

        - MPE-20

        - MPE-40

        - MPE-CAPS

- Manifold:

- MPM-050

- MPE carries a 2-year warranty.