HUNTER PGJ

PROFESSIONAL SERIES
ADJUSTABLE-ARC 
GEAR-DRIVEN SPRINKLER

The sprinkler shall be of the gear-driven, rotary type, capable of covering a _________ foot (meter) radius at _________ PSI (bars; kPa) with a discharge rate of _________ GPM (m3/hr; l/m). The sprinkler shall be available with eight (8) nozzles discharging from .64 to 5.3 GPM (0 .15 to 1.2 m3/hr; 2.4 to 20.1 l/m). The sprinkler shall have radius reduction capabilities by means of a stainless-steel nozzle retainer/radius adjustment screw. Adjustments of the nozzle retainer/radius adjustment screw shall be done using a 3/32” hex wrench or small flat-blade screwdriver.

The sprinkler shall be available in an adjustable part-circle configuration. The adjustable part-circle unit shall be minutely adjustable from 40° to 360°. The adjustable unit shall be adjustable in all phases of installation (i.e., before installation, after installation while static, and after installation while in operation). Arc adjustments shall be done using a tool specific to the sprinkler to prevent unauthorized modifications to the arc pattern.

The sprinkler shall have a 4-inch (10 cm), 6-inch (15 cm), or 12-inch (30 cm) pop-up stroke, depending on the body specified, to bring the rotating nozzle turret into a clean environment. The sprinkler shall be available as an aboveground shrub head. The pop-up versions of the sprinkler shall be available with a drain check valve to prevent low head drainage, and be capable of checking up to 7 feet (2.1 m) in elevation change. When specified, the sprinkler shall have a cover molded of purple Alcryn rubber to indicate the use of reclaimed water and come equipped with a drain check valve.

The sprinkler shall have an exposed surface diameter after installation of 1 1/4 inches (3 cm), and have an overall height of 6-1/2 inches (17 cm) (shrub), 6-5/8 inches (17 cm) on the 4-inch pop-up, 8-5/8 inches (22 cm) on the 6-inch pop-up, 15-7/8 inches (40 cm) on the 12-inch pop-up. The sprinkler shall have a 1/2-inch Female National Pipe Thread (FNPT) inlet. The sprinkler shall be serviceable after installation by unscrewing the body cap, removing the riser assembly, and extracting the inlet filter screen.

The body and riser of the sprinkler shall be constructed of corrosion resistant, impact resistant, heavy-duty A.B.S. It shall have a stainless steel spring for positive retraction of the riser when irrigation is complete. The sprinkler shall carry a two-year, exchange warranty (not prorated).

The sprinkler shall be manufactured by Hunter Industries Incorporated, San Marcos, California.