- The Inline Emitter Tubing shall be Hunter PROFESSIONAL LANDSCAPE DRIP LINE (PLD).

The inline emitter shall be welded to the inner circumference of the polyethylene tubing. The inline emitter shall have dual outlet ports, 180° apart, ensuring only one port has contact with the ground when the tubing is installed at grade and mulched over.

Hunter’s PROFESSIONAL LANDSCAPE DRIP LINE (PLD) emitter shall pressure compensate by lengthening the emitter’s turbulent flow path. The emitter shall be cylindrical in shape and provide surface area for filtration throughout 360° of its outer circumference.  This increased filtration surface area shall assure that the water that enters the inline emitter can always come from the upper half, or cleanest part of the flow path in the polyethylene tubing regardless of how the inline tubing lays on the ground.

Hunter Professional Drip tubing shall be brown in color and conform to a Nominal .5575” Inside Diameter, Nominal .045” Wall Thickness and shall be compatible with all industry standard 17MM fittings (Both barbed and compression) Hunter Professional Drip tubing shall have factory installed, check valve and pressure-compensating, inline emitters installed every [12] or [18] or [24] inches (30, 45, or 60 cm). OR

Hunter Professional Drip tubing shall have factory installed, check valve and pressure-compensating, inline emitters with spacing as indicated on drawings. When specified the tubing shall be supplied without any emission devices inserted (PLDBLNK).

The flow rate from each installed inline emitter shall be a consistent [0.4], [0.6] or [1.0] gallons per hour (1.35, 2.35, or 3.75 liters per hour [LPH] )or when inlet pressure is between 15 and 50 psi (1.0 to 3.5 bar).

OPERATIONAL PRESSURE RANGE OF 15-50PSI (1.0-3.5 bar)

Models:

Check Valve and Pressure-Compensating Professional Landscape Drip line for potable water systems:

Hunter PROFESSIONAL LANDSCAPE DRIP LINE (PLD) with emitters and spacing as shown on construction drawings.

- Hunter Professional Drip Tubing PLD-04-12 (0.4GPH emitters spaced 12 inches on-center) (1.35 LPH emitters spaced 30 cm on-center)

- Hunter Professional Drip Tubing PLD-04-18 (0.4GPH emitters spaced 18 inches on-center) (1.35 LPH emitters spaced 45 cm on-center)

- Hunter Professional Drip Tubing PLD -04-24 (0.4GPH emitters spaced 24” on-center) (1.35 LPH emitters spaced 60 cm on-center)

- Hunter Professional Drip Tubing PLD -06-12 (0.6GPH emitters spaced 12” on-center) (2.35 LPH emitters spaced 30 cm on-center)

- Hunter Professional Drip Tubing PLD -06-18 (0.6GPH emitters spaced 18” on-center) (2.35 LPH emitters spaced 45 cm on-center)

- Hunter Professional Drip Tubing PLD -06-24 (0.6GPH emitters spaced 24” on-center) (2.35 LPH emitters spaced 60 cm on-center)

- Hunter Professional Drip Tubing PLD -10-12 (1.0 GPH emitters spaced 12” on-center) (3.75 LPH emitters spaced 30 cm on-center)

- Hunter Professional Drip Tubing PLD -10-18 (1.0 GPH emitters spaced 18” on-center) (3.75 LPH emitters spaced 45 cm on-center)

- Hunter Professional Drip Tubing PLD -10-24 (1.0 GPH emitters spaced 24” on-center) (3.75 LPH emitters spaced 60 cm on-center)

PLD shall carry a two-year, exchange warranty (not prorated).

The Landscape Drip Line shall be manufactured by Hunter Industries Incorporated, San Marcos, California.